## STANDARD OPERATING PROCEDURE (SOP)

### TITLE:

### Preparation of Giemsa Stock Solution

### 1. PURPOSE

To provide a standardized procedure for the preparation of Giemsa stock solution used for staining blood films for malaria parasites, blood cells, and other hematological or parasitological examinations.

### 2. SCOPE

This SOP applies to all Medical Laboratory Scientists and Technicians involved in blood film staining within the Haematology and Parasitology units.

### 3. RESPONSIBILITY

- Laboratory Scientists/Technicians: Preparation and proper storage

- Unit Head/Supervisor: Quality control and compliance

### 4. PRINCIPLE

Giemsa stain is a Romanowsky stain containing methylene blue, eosin, and azure dyes. It stains nuclei, cytoplasm, and parasites differentially, allowing clear visualization of blood cells and malaria parasites.

### 5. MATERIALS AND REAGENTS

- Giemsa powder (certified)

- Absolute methanol (analytical grade)

- Glycerol (AR grade)

- Measuring cylinder

- Amber-colored reagent bottle

- Glass rod or magnetic stirrer

- Water bath

- Label and marker

### 6. PROCEDURE (STOCK SOLUTION PREPARATION)

- Weigh 3.8 g of Giemsa powder accurately.

- Transfer into a clean, dry container.

- Add 250 mL of glycerol.

- Warm gently in a water bath at 55–60°C for about 1–2 hours, stirring occasionally until fully dissolved.

- Remove from heat and allow to cool.

- Add 250 mL of absolute methanol slowly while mixing.

- Mix thoroughly to ensure homogeneity.

- Transfer into an amber reagent bottle.

- Label with:

  - Name of reagent

  - Date of preparation

  - Expiry date

  - Prepared by

### 7. STORAGE AND STABILITY

- Store at room temperature (20–25°C)

- Keep tightly closed and protected from light

- Allow to mature for at least 1 week before use

- Stable for 6–12 months if properly stored

### 8. QUALITY CONTROL

- Test stain on a known positive malaria blood film

- Ensure:

  - RBCs appear pale pink

  - Nuclei stain purple

  - Malaria parasites stain blue-purple

- Reject stain if precipitate forms or staining quality deteriorates

### 9. SAFETY PRECAUTIONS

- Methanol is toxic and flammable – handle in a well-ventilated area

- Wear gloves and lab coat

- Avoid inhalation and skin contact

- Dispose of waste according to laboratory safety policy

### 10. REFERENCES

- Cheesbrough M. District Laboratory Practice in Tropical Countries, Part 1

- WHO. Basic Malaria Microscopy

- Dacie and Lewis. Practical Haematology